CashCode FL BDP

Interface

(WBA Compatible)

for Bill Validator

Interface Descriptions

1.	Introduction	4

2.	Communication format	5

2.1. Common message's format	5

2.1.2.	Controller's message set:	5

2.1.3.	Acceptor's message set:	5

2.2. Transmission specifications	6

3.	Communication flow	7

Transmission of status request	7

3.2. Transmission command to Acceptor	7

Communication error I	7

3.4. Communication error II	8

3.5. Communication error III	8

Communication error IV	8

4.	Command/response description	9

4.1. Status	9

4.1.2.	STATUS REQUEST	9

4.1. Operation commands	12

4.1.2.	RESET	12

4.1.3.	STACK-1	12

4.1.4.	STACK-2	12

4.1.5.	RETURN	12

4.1.6.	HOLD	12

4.1.7.	WAIT	13

4.2. Response to [VEND VALID]	13

4.2.2.	ACK (Affirmative response)	13

4.3. Setting commands	13

4.3.2.	ENABLE/DISABLE	13

4.3.3.	SECURITY	14

4.3.4.	INHIBIT	14

4.3.5.	DIRECTION	14

4.3.6.	OPTIONAL FUNCTION	15

4.4. Setting status request	15

4.4.2.	ENABLE/DISABLE	15

4.4.3.	SECURITY	15

4.4.4.	INHIBIT	16

4.4.5.	DIRECTION	16

4.4.6.	VERSION REQUEST	17

4.4.7.	BOOT VERSION REQUEST	17

4.4.8.	OPTIONAL FUNCTION	18

5.	Timing chart	19

5.1. Power ON	19

5.2. Accepting of bill	21

5.3. Rejection of bills  by validating	24

5.4. Returning of bill by [RETURN] command	25

5.5. Inhibit of accepting by Acceptor	26

5.6. Stacker full (STACK-1)	27

5.7. Bill jamming at the time of returning	28

6.	Attachment 1. Command/response list	29

# Introduction

This specification describes specifications at the data level of interfacing between the Acceptor and Controller. In regard to electrical connections and operation, refer to the specification manuals for each model.

FL BDP interface is 2-way serial interface. Control over the status and of the Acceptor is made by polling [STATUS REQUEST] and by commands ([OPERATION COMMAND] [SETTING COMMANDS]) from the Controller and also setting and confirming of functions can be made.

# Communication format

## Common message's format

|  |  | FCh |  | LNG |  | CMD |  | DATA |  | CRC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LNG : |  | 1 byte |  |  |  | Data length (Total number of bytes of entire message) or 0x05h if no data |  |  |
|  |  |  | C/R : |  | 1 byte |  |  |  | Command or response (status) |  |  |
|  |  |  | DATA: |  | 0 to 250 byte |  |  |  | Data if |  |  |
|  |  |  | CRC: |  | 2 byte |  |  |  | Check code by CRC method |  |  |
|  |  |  |  |  |  |  | DATA |  | Object section is from message's begin to end of DATA |  |  |
|  |  |  |  |  |  |  |  |  | (Initial value = 0) |  |  |
|  | Error control method |  |  |  |  |  |  |  | Error detection CRC method |  |  |
|  |  |  |  |  |  |  |  |  | CRC - CCITT p(x) = X14 + X12 + X5 + l |  |  |

### Controller's message set:

Polling (status request)

| FCh | 05h | 11h | 2756h |
| --- | --- | --- | --- |

Acknowledgement (ACK)

| FCh | 05h | 50h | AA05h |
| --- | --- | --- | --- |

Command

| FCh | LNG | C/R | DATA | CRC |
| --- | --- | --- | --- | --- |

LNG :	Data length

C/R :	Command

DATA :	Command's data

CRC :	Check code by CRC method

### Acceptor's message set:

Acknowledgement (ACK)

| FCh | 05h | 50h | AA05h |
| --- | --- | --- | --- |

Response format I

| FCh | LNG | C/R | CRC |
| --- | --- | --- | --- |

LNG :	Data length

C/R :	Status code, returned from acceptor

CRC :	Check code by CRC

Response format II

| FCh | LNG | C/R | DATA | CRC |
| --- | --- | --- | --- | --- |

LNG :	Data length

C/R :	Response

DATA :	Response's Data

CRC :	Check code by CRC method

## Transmission specifications

| Transmission method | Full duplex transmission |  |
| --- | --- | --- |
| Transmission speed | 9600 bps/19200 bps |  |
|  | (Depending on the model, setting by DIP switches is |  |
|  | possible.) |  |
| Synchronizing method | Asynchronous method |  |
| Connection control method | Polling method |  |
| Data format | Start bit | 1 |
|  | Data bit | 8 |
|  | Parity bit | EVEN |
|  | Stop bit | 1 |
|  | X parameter | None |

# Communication flow

Controller

Acceptor

Status request

Status

Status request

Status

Less or equal to 50ms

From 100 to 200 ms

Transmission of status request

## Transmission command to Acceptor

Controller

Acceptor

Status request

Status

Less or equal to 50ms

Communication error I

Controller

Acceptor

Status request

Less or equal to 50ms

Status

No response

Status request

Status

No response

3 seconds and no response

(Failure of communication system and power source OFF, failure etc. of acceptor.)

## Communication error II

Controller

Acceptor

No data in 200ms

Status request

No reception

Status request

No reception

(Failure etc. of communication system)

## Communication error III

Controller

Acceptor

Status request

Communication error

CRC error, loss of data, etc. occurs

Status request

Communication error

Controller

Acceptor

Operation command

Invalid command

Operation command

Invalid command

Improper command in processing action of acceptor

Communication error IV

# Command/response description

## Status

### STATUS REQUEST

- Request from Controller to Acceptor on status of Acceptor. By this command, the Controller monitors operation status, resetting from error status etc. of the Acceptor.

- Response from Acceptor expresses the present status of the Acceptor which is normally divided into normal status, power-up status and error status.

- By the power supply ON status, the Acceptor returns the following status. Also, this status is held until a [RESET] command is sent from the Controller. (Refer to 7-2).

- Method for releasing error status differs by model. For details, refer to the [Specification manual] and [DATA] setting specification manual] by model.

Controller's command code: 11h.

Acceptor's response: one from the following.

| Status |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11h | ENABLE (IDLING) | Bill accepting standby status and operation able status. |  |  |
| 12h | ACCEPTING | Status in which bills are taken in and validating is being conducted. |  |  |
| 13h + Data | ESCROW | Status in which bill validation is completed and command from the Controller is awaited (The bill is held inside the Acceptor) [ESCROW DATA] (accepted denomination) of 1 byte is added. If [STATUS REQUEST] cannot be received from the Acceptor within 3 seconds while in the [ESCROW] status, and when an operation command from the Controller is not transmitted within 10 second after transmission, the bill is returned. ESCROW DATA (Accepted denomination) |  |  |
|  |  | DATA |  | Denomination |
|  |  | 61h |  | 01 |
|  |  | 62h |  | 02 |
|  |  | 63h |  | 03 |
|  |  | 64h |  | 04 |
|  |  | 65h |  | 05 |
|  |  | 66h |  | 06 |
|  |  | 67h |  | 07 |
|  |  | 68h |  | 08 |
|  |  | *) The accepted denominations are described in the [DATA setting specifications] for each model. |  |  |
| 14h | STACKING | Status in which bills are conveyed to the stacker and stored in accordance with OPERATION COMMAND's [STACK-1] and [STACK-2] from the Controller. (Refer to 6-3. 7-2) |  |  |
| 15h | VEND VALID | Confirming signal of bill acceptance. Against [VEND VALID], the Acceptor holds its status until [ACK] is sent from the Controller. The Controller conducts credit up by [VEND VALID]. (Refer to 7-2) |  |  |
| 16h | STACKED | Status from; the time bill is stored up to the time accepting of the next bill from [VEND VALID] becomes possible ([ENABLE] status) |  |  |
| 17h + DATA | REJECTING | A status in which unacceptable bills as the result of bill validating by the Acceptor or bills by an [INHIBIT] command from the Controller are returned. (Refer to 7-3)'. [REJECT DATA] (description of rejection) of 1 byte is added. REJECT DATA (Rejection description) |  |  |
|  |  | DATA | Description |  |
|  |  | 71h | Insertion error |  |
|  |  | 72h | Mag error |  |
|  |  | 73h | Rejection action by remaining of bills etc. (Acceptor head section) |  |
|  |  | 74h | Compensation error/multiplying factor error |  |
|  |  | 75h | Conveying error |  |
|  |  | 76h | Denomination assessing error |  |
|  |  | 77h | Photo pattern error  |  |
|  |  | 7Sh | Photo level error |  |
|  |  | 79h | Return by inhibit/insertion direction, denomination error in case a command against escrow is not transmitted |  |
|  |  | 7Ah |  |  |
|  |  | 7Bh | Operation error |  |
|  |  | 7Ch | Rejecting action by remaining of bills and such (stacker section) |  |
|  |  | 7Dh | Length error |  |
|  |  | 7Eh | Photo pattern error  |  |
| 18h | RETURNING | Against [ESCROW], a status in which a bill is returned by a [RETURN] command from the Controller. (Refer to 7-4) |  |  |
| 19h | HOLDING | Against [ESCROW], a status in which a bill is held inside the Acceptor by a [HOLD] command from the Controller. |  |  |
| 0Ah | DISABLE (INHIBIT) | A status in which acceptance of bills by the Acceptor is inhibited by a [INHIBIT] command from the Controller. (Refer to 7-5) Also a status in which all acceptable denominations are in a disable status by [ENABLE/DISABLE] command or where all receiving directions are in an inhibit status by a [DIRECTION] command. |  |  |
| 1Bh | INITIALIZE | Status in which the Acceptor is conducting initializing action by [RESET] from the Controller. The setting command from the Controller is effective only in this status. (Refer to 7-1) |  |  |
| Power Up Status |  |  |  |  |
| 40h | POWER UP | A status in which status inside the Acceptor is normal with the Acceptor power on. |  |  |
| 41h | POWER UP with BILL IN Acceptor | A status in which bills remain in the Acceptor head conveying section (a return possible position) with the power supply on. By a [RESET] command from the Controller, the Acceptor returns the bill and conducts initializing. |  |  |
| 42h | POWER UP WITH BILL IN STACKER | A status in which bills remain in the stacker conveying section (a return possible position) with the power supply on. By a [RESET] command from the Controller, the Acceptor stores the bill and conducts initializing. |  |  |
| Error Status |  |  |  |  |
| 43h | STACKER FULL | A stacker box full condition (Refer to 7-6) |  |  |
| 44h | STACKER OPEN (STACKER BOX REMOVE) | The stacker door is open or the stacker box is not mounted. |  |  |
| 45h | JAM IN Acceptor | Jamming has occurred inside the Acceptor |  |  |
| 46h | JAM IN STACKER | Jamming has occurred in the stacker conveying section. An abnormal condition has developed at the tine of storing. |  |  |
| 47h | PAUSE | A condition, which the Acceptor cannot operate because a second bill has been inserted while the first bill is being stored or conveyed (when the second bill is removed, conveying is started). |  |  |
| 48h | CHEATED | An action thought to be mischievous against the Acceptor has been committed. |  |  |
| 49h + DATA | FAILURE | A status in which normal operation cannot be made because of a failure, an abnormal condition, or incorrect setting of the Acceptor. FAILURE DATA (abnormal contents) |  |  |
|  |  | DATA | Contents |  |
|  |  | A2h | Stack motor failure |  |
|  |  | A5h | Transport (feed) motor speed failure |  |
|  |  | A6h | Transport (feed) motor failure |  |
|  |  | ABh | Cash box not ready |  |
|  |  | AFh | Validator head remove |  |
|  |  | BOh | BOOT ROM failure |  |
|  |  | B1h | External ROM failure |  |
|  |  | B2h | ROM failure |  |
|  |  | B3h | External ROM writing failure |  |
| 4Ah | COMMUNICATION ERROR | An error has developed in the communication data. (Refer to Communication error III) |  |  |
| 4Bh | INVALID C0MMAND | Command from the Controller is not valid. (Refer to Communication error IV) |  |  |

## Operation commands

### RESET

Controller's command code: 40h.

Acceptor's response: ACK.

A command for resetting the Acceptor.  The Acceptor accepts this command regardless of its status. After the power supply is turned on (power up status), transmission is required without fail.

### STACK-1

Controller's command code: 41h.

Acceptor's response: ACK in [ESCROW] status; otherwise [INVALID COMMAND] 4Bh.

A bill in an escrow status is conveyed to the stacker section and stored. The Acceptor becomes in a [VEND VALID] status when the bill passes the stacker lever.

*) The position of STACK-1 may differ by model.

### STACK-2

Controller's command code: 42h.

Acceptor's response: ACK in [ESCROW] status; otherwise [INVALID COMMAND] 4Bh.

A bill in escrow status is conveyed to the stacker and stored. The Acceptor becomes in a [VEND VALID] status when a bill is stored (pushed in position).

*) The position of STACK-2 may differ by model.

### RETURN

Controller's command code: 43h.

Acceptor's response: ACK in [ESCROW] status; otherwise [INVALID COMMAND] 4Bh.

Returns a bill in an escrow status.

### HOLD

Controller's command code: 44h.

Acceptor's response: ACK in [ESCROW] status; otherwise [INVALID COMMAND] 4Bh.

A bill in escrow status is made to be held for 10 seconds. For continued holding, resenting of a [HOLD] command is necessary.

### WAIT

Controller's command code: 45h.

Acceptor's response: ACK.

Status of Acceptor is made to be held for 2 seconds. To continue holding this status, resenting of a [WAIT] command is necessary.

## Response to [VEND VALID]

### ACK (Affirmative response)

Code: 50h

Acceptor's response against an [OPERATION COMMAND] from the Controller.

Controller's response against [VEND VALID] from the Acceptor.

## Setting commands

### ENABLE/DISABLE

Controller's command and data:

| C0h |  | + | DATA1 |  |  | + | DATA2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATA1 DATA2 |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 08 | 07 |  |  | 06 | 05 |  | 04 | 03 | 02 | 01 | <- Denomination |
|  |  |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 |  |

0:enable

1:disable (default: 00h)

Acceptor's response:

| C0h | + | DATA1 | + | DATA2 | [echo back] |
| --- | --- | --- | --- | --- | --- |

Accepting by each denomination is set.

[ENABLE/DISABLE DATA] of 2 bytes are added. Receiving is possible only then Acceptor is initialized. However INHIBIT is excluded.

### SECURITY

Controller's command and data:

| C1h | + | DATA1 |  |  | + | DATA2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA1 DATA2 |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 08 | 07 |  |  | 06 | 05 |  | 04 | 03 | 02 | 01 | <- Denomination |
|  |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 |  |

0: normal; 1: security level high  (default: 00h)

Acceptor's response:

| C1h | + | DATA1 | + | DATA2 | [echo back] |
| --- | --- | --- | --- | --- | --- |

Validating level by each denomination is set. [SECURITYDATA] of 2 bytes is added. Receiving is possible only then Acceptor is initialized. However INHIBIT is excluded.

### INHIBIT

Controller's command and data:

| C3h | + | DATA(1byte) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 0 | 0 |  | 0 | 04 | 03 | 02 | 01 |  |

0: not inhibit

1: inhibit      (default: 01h)

Acceptor's response:

| C3h | + | DATA(1byte) | [echo back] |
| --- | --- | --- | --- |

Status of Acceptor is temporarily made acceptance inhibit. [INHIBIT DATA] of 1 byte are added.

* INHIBIT can be accepted in any status.

| Set during acceptance of bill | Return bill and becomes in INHIBIT status |
| --- | --- |
| Set during validating of bill |  |
| Set while in escrow status |  |
| Set during storing of bill | After storing bill. becomes in INHIBIT status. |
| Set while in vend valid |  |

### DIRECTION

Controller's command and data:

| C4h | + | DATA(1byte) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  | <- Direction |
|  |  |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | inh |  |

0: not inhibit

1: inhibit

Acceptor's response:

| C4h | + | DATA(1byte) | [echo back] |
| --- | --- | --- | --- |

Sets accepting direction of bill.

[DIRECTION DATA] of 1 byte are added.

### OPTIONAL FUNCTION

Controller's command and data:

| C5h | + | DATA1 |  |  | + | DATA2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA1 DATA2 |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 08 | 07 |  |  | 06 | 05 |  | 04 | 03 | 02 | 01 | <- OPTION |
|  |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 |  |

0: disable

1: enable (default: 00h)

Acceptor's response:

| C5h | + | DATA1 | + | DATA2 | [echo back] |
| --- | --- | --- | --- | --- | --- |

Sets option function of Acceptor.

[OPTIONAL FUNCTION DATA] of 2 bytes is added.

## Setting status request

### ENABLE/DISABLE

Controller's command and data:

| 80h |  | + | DATA1 |  |  | + | DATA2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DATA1 DATA2 |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 08 | 07 |  |  | 06 | 05 |  | 04 | 03 | 02 | 01 | <- Denomination |
|  |  |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 |  |

0: enable

1: disable (default: 00h)

Acceptor's response:

| 80h | + | DATA1 | + | DATA2 | [echo back] |
| --- | --- | --- | --- | --- | --- |

Transmission request for set acceptance status of acceptor by each denomination [ENABLE/DISABLE] commands as well as status of accepting denominations set by DIP switches are added as [ENABLE/DISABLE DATA] of 2 bytes.

*) Settings of DIP switches are described in the [Specification manual] for each model.

### SECURITY

Controller's command and data:

| 81h | + | DATA1 |  |  | + | DATA2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA1 DATA2 |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 08 | 07 |  |  | 06 | 05 |  | 04 | 03 | 02 | 01 | <- Denomination |
|  |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 |  |

0: normal; 1: security level high  (default: 00h)

Acceptor's response:

| 81h | + | DATA1 | + | DATA2 | [echo back] |
| --- | --- | --- | --- | --- | --- |

Request for transmission of set status of validating level by each denomination

[SECURITY DATA] of 2 bytes are added.

### INHIBIT

Controller's command and data:

| 83h | + | DATA(1byte) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 0 | 0 |  | 0 | 04 | 03 | 02 | 01 |  |

0: not inhibit

1: inhibit      (default: 01h)

Acceptor's response:

| 83h | + | DATA(1byte) | [echo back] |
| --- | --- | --- | --- |

Transmission request for set status of acceptance inhibit of the Acceptor

[INHIBIT DATA] of 1 byte are added.

### DIRECTION

Controller's command and data:

| 84h | + | DATA(1byte) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  | <- Direction |
|  |  |  | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | inh |  |

0: not inhibit

1: inhibit

Acceptor's response:

| 84h | + | DATA(1byte) | [echo back] |
| --- | --- | --- | --- |

Request for transmission of set status of bill acceptance direction.

[DIRECTION DATA] of 1 byte are added

### VERSION REQUEST

Controller's command code: 88h.

Acceptor's response:

| 88h | + | DATA |
| --- | --- | --- |

DATA

The Acceptor responds to MODEL/ID/VERS10N/CRS etc. by ASCII data.

Data length expresses the following meanings from the top, as

x  (xxx)  xx  xx  ID003-xx  xxx-xx  ddmmmyy  xxxx

CRC check code

Date May. 1. 1999 ==> “01may99”

Software version

Interface type

Stacker type

Model No.

Country code

Model code

([LNG]-5) bytes (variable).

Request for transmission of Acceptor MODEL/ID/VERSION

ASCII data of 36 bytes are added

### BOOT VERSION REQUEST

Controller's command code: 89h.

Acceptor's response:

| 89h | + | DATA(4byte) |
| --- | --- | --- |

DATA

The Acceptor responds to the BOOT VERSION by 4 byte ASCII data.

BOOT version

“Bxx"

Request for of BOOT VERSION of Acceptor

ASCII data of 4 bytes are added.

### OPTIONAL FUNCTION

Controller's command and data:

| 85h | + | DATA1 |  |  | + | DATA2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATA1 DATA2 |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 08 | 07 |  |  | 06 | 05 |  | 04 | 03 | 02 | 01 | <- OPTION |
|  |  |  | bit7 bit6 bit5 bit4 bit3 bit2 bit1 bit0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 | 0 | 0 |  |

0: disable

1: enable (default: 00h)

Acceptor's response:

| 85h | + | DATA1 | + | DATA2 | [echo back] |
| --- | --- | --- | --- | --- | --- |

Request for transmission of setting status of [OPTIONAL FUNCTION] command.

[OPTIONAL FUNCTION DATA] of 2 bytes are added

# Timing chart

## Power ON

From charging of power supply to standby status

Status

Controller				 Acceptor

[STATUS REQUEST] [STATUS REQUEST] [STATUS REQUEST] [POWER UP] [VERSION REQUEST] REQUEST] [VERSION INFORMATION] [RESET] [ACK] [INITIALIZE] [STATUS REQUEST] [ENABLE/DISABLE] [ENABLE/DISABLE] [SECURITY] [SECURITY] [OPTIONAL FUNCTION] [OPTIONAL FUNCTION] [INHIBIT] [INHIBIT] [INITIALIZE] [STATUS REQUEST] [ENABLE/](IDLING) [STATUS REQUEST] Power supply charged (SETTING REQUEST) (OPERATION COMMAND) (SETTING COMMAND) (SETTING COMMAND) (SETTING COMMAND) (SETTING COMMAND) Power supply cut cut

Power supply on

Initializing

On standby

From charging of power supply to standby status

When bill remains inside the Acceptor at the time of power charging

Controller			Acceptor

Status

[STATUS REQUEST]

[STATUS REQUEST]

[STATUS REQUEST]

[POWER UP WITH BILL

IN Acceptor]

[RESET]

[ACK]

[INITIALIZE]

[STATUS REQUEST]

[STATUS REQUEST]

[INITIALIZE]

[STATUS REQUEST]

[INITIALIZE]

[ENABLE/](IDLING)

[STATUS REQUEST]

(OPERATION COMMAND)

[POWER UP WITH BILL

IN STACKER]

Bill remains

In a return possible position

(Bill remains

in a return impossible

Bill is returned (stored) by

initializing by [RESET]

command

Power supply Power supply charged cut

Power supply on

Initializing

On standby

## Accepting of bill

Accepting of bill by [STACK –1] command

Controller			Acceptor

Status

[STATUS REQUEST]

[ENABLE](IDLING)

[STATUS REQUEST] REQUEST]

[ACCEPTING]

[STATUS REQUEST]

[ESCROW]

[ACK]

[STACK-1]

[STATUS REQUEST]

[STACKING]

[STATUS REQUEST]

[VEND VALID]

[STATUS REQUEST]

[STACKED]

[STACKED]

[STATUS REQUEST]

[ENABLE/](IDLING)

[STATUS REQUEST]

[STATUS REQUEST]

[ACCEPTING]

(OPERATION COMMAND)

[ACK]

Bill is returned if no

[STATUS REQUEST] is

made within 3 seconds

while  in ESCROW status

Bill is returned if no

[OPERATION COMMAND]

is  made within 10 seconds

against [ESCROW]

in ESCROW status

On stanby Bill insertion

Validating

Escrow

Feed

Vend valid

Storing

On standby

Accepting of bill by [STACK –2] command

Controller			Acceptor

Status

[STATUS REQUEST]

[ENABLE](IDLING)

[STATUS REQUEST] REQUEST]

[ACCEPTING]

[STATUS REQUEST]

[ESCROW]

[ACK]

[STACK-2]

[STATUS REQUEST]

[STACKING]

[STATUS REQUEST]

[STACKING]

[STATUS REQUEST]

[VEND VALID]

[STACKED]

[STATUS REQUEST]

[ENABLE/](IDLING)

[STATUS REQUEST]

[STATUS REQUEST]

[ACCEPTING]

(OPERATION COMMAND)

[ACK]

On stanby Bill insertion

Validating

Escrow

Feed

Storing

Vend valid

Storing

On standby

Resenting of [VEND VALID]

[STATUS REQUEST] REQUEST]

[ACCEPTING]

[STATUS REQUEST]

[ESCROW]

[ACK]

[STACK-1]

[STATUS REQUEST]

[STACKING]

[STATUS REQUEST]

[VEND VALID]

[STATUS REQUEST]

[STACKED]

[STACKED]

[STATUS REQUEST]

[STATUS REQUEST]

[ACCEPTING]

(OPERATION COMMAND)

[ACK]

Resenting of [VEND VALID]

Against [VEND VALID],

Status is held until.

[ACK] is send

[STATUS REQUEST]

[VEND VALID]

[ACK]

NO RESPONSE

Valdating

Escrow

Feed

Vend valid

Storing

## Rejection of bills  by validating

Controller			Acceptor

Status

[STATUS REQUEST] [ENABLE](IDLING) [STATUS REQUEST] REQUEST] [ACCEPTING] [STATUS REQUEST] [REJECTING] [REJECTING] [STATUS REQUEST] [ENABLE/](IDLING) [STATUS REQUEST] [STATUS REQUEST] [ACCEPTING] On stanby Bill insertion

Validating

Returning

On standby

## Returning of bill by [RETURN] command

Controller			Acceptor

Status

[STATUS REQUEST]

[ENABLE](IDLING)

[STATUS REQUEST] REQUEST]

[ACCEPTING]

[STATUS REQUEST]

[ESCROW]

[ACK]

[RETURN]

[STATUS REQUEST]

[RETURNING]

[STATUS REQUEST]

[RETURNING]

[STATUS REQUEST]

[ENABLE/](IDLING)

[STATUS REQUEST]

[ACCEPTING]

(OPERATION COMMAND)

On stanby Bill insertion

Validating

Escrow

Returning

On standby

## Inhibit of accepting by Acceptor

Controller			Acceptor

Status

[STATUS REQUEST]

[ENABLE](IDLING)

[INHIBIT]

[INHIBIT]

[STATUS REQUEST]

[DISABLE](INHIBIT)

[STATUS REQUEST]

[DISABLE](INHIBIT)

[STATUS REQUEST]

[ENABLE](IDLING)

(OPERATION COMMAND)

(SETTING STATUS)

On stanby

Inhibit

## Stacker full (STACK-1)

Status

Controller			Acceptor

[STATUS REQUEST]

Bill insertion [ENABLE](IDLING) [VEND VALID] [ACK] [STATUS REQUEST] [STACKED] [STACKED] [STATUS REQUEST] [STACKED FULL] [STATUS REQUEST] [STATUS REQUEST] [ACCEPTING] [STATUS REQUEST] [STACKED FULL] [INITIALIZE] [STATUS REQUEST] [INITIALIZE] [STATUS REQUEST] [DISABLE](INHIBIT) [STATUS REQUEST] Extracting of bill Validating On stanby

Vend valid

Storing

Initializing

On standby

## Bill jamming at the time of returning

Controller			Acceptor

Status

Bill insertion [STATUS REQUEST] [ENABLE](IDLING) [STATUS REQUEST] REQUEST] [ACCEPTING] [STATUS REQUEST] [REJECTING] [REJECTING] [STATUS REQUEST] [STATUS REQUEST] [REJECTING] [STATUS REQUEST] [REJECTING] [STATUS REQUEST] [JAM IN Acceptor] [JAM IN Acceptor] [STATUS REQUEST] [DISABLE](INHIBIT) [STATUS REQUEST] [STATUS REQUEST] [ACCEPTING] Removing of jammed bills On stanby Bill jamming occurs

Validating

Returning

F A I L

On standby

# Attachment 1. Command/response list

| Controller ==> Acceptor |  | Acceptor ==> Controller |  |  |
| --- | --- | --- | --- | --- |
| STATUS |  |  |  |  |
| STATUS REQUEST | 11H | ENABLE (IDLING) |  | 11H |
|  |  | ACCEPTING |  | 12H |
|  |  | ESCROW |  | 13H+DATA |
|  |  | STAKING |  | 14H |
|  |  | VEND VALID |  | 15H |
|  |  | STACKED |  | 16H |
|  |  | REJECTING |  | 17H+DATA |
|  |  | RETURNING |  | 18H |
|  |  | HOLDING |  | 19H |
|  |  | DISABLE (INHIBIT) |  | 1AH |
|  |  | INITIALIZE |  | 1BH |
|  |  | POWER UP STATUS |  |  |
|  |  | POWER UP |  | 40H |
|  |  | POWER UP WITH BILL IN ACCEPTOR |  | 41H |
|  |  | POWER UP WITH BILL IN STACKER |  | 42H |
|  |  | ERROR STATUS |  |  |
|  |  | STACKER FULL |  | 43H |
|  |  | STACKER OPEN |  | 44H |
|  |  | JAM IN ACCEPTOR |  | 45H |
|  |  | JAM IN STACKER |  | 46H |
|  |  | PAUSE |  | 47H |
|  |  | CHEATED |  | 48H |
|  |  | FAILURE |  | 49H+DATA |
|  |  | COMMUNICATION ERROR |  | 4AH |
| RESPONSE TO [VEND VALID] |  |  |  |  |
| ACK | 50H |  |  |  |
| OPERATION COMMAND |  | RESPONSE TO OPERATION COMMAND |  |  |
| RESET STACK-1 STACK-2 RETURN HOLD WAIT | 40H 41H 42H 43B 44H 45H | ACK INVALID COMMAND |  | 50H 4BH |
| SETTING COMMAND |  | RESPONSE TO SETTING COMMAND |  |  |
| ENABLE/DISABLE (DENOMI SECURITY (DENOMI) INHIBIT (ACCEPTOR) DIRECTION OPTIONAL FUNCTION | COH+DATA C1H+DATA C3H+DATA C4H+DATA C5H+DATA | ENABLE/DISABLE (DENOIII) : SECURITY (DENOMI) INHIBIT (ACCEPTOR) DIRECTION OPTIONAL FUNCTION ; |  | C0H+ DATA C1H+DATA C3H+DATA C4H+DATA C5H+DATA |
| SETTING STATUS REQUEST |  | SETTING STATUS |  |  |
| ENABLE/DISABLE (DENOMI) SECURITY (DENOMI) INHIBIT (ACCEPTOR) DIRECTION OPTIONAL FUNCTION VERSION REQUEST BOOT VERSION REQUEST | 80H 81H 83H 84H 85H 88H 89H | ENABLE/DISABLE (DENOMI) SECURITY (DENOMI) INHIBIT (ACCEPTOR) DIRECTION OPTIONAL FUNCTION VERSION INFORMATION BOOT VERSION INFORMATION |  | 80H+data 81H+DATA 83H+DATA 84H+DATA 85H+DATA 88H+DATA 89H+DATA |